班級：　四電子一乙　　　　　　　     姓名：李家睿　　　　　      學號：B11102112

Binary to Decimal converter

- Design

- Schematic entry

- full adder:

    - Block diagram

    - Behavioral

    - Timing verification

- Geq5Plus3

    - Block diagram

    - Behavioral

    - Timing verification

- Binary-to-BCD converter :

    - Block diagram

    - Behavioral

    - Timing verification

- Verilog-HDL-entry

- Verilog entry

| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 | module binarytobcd( input clk, input [7:0] binary_value, output reg [3:0] one = 0, output reg [3:0] ten = 0, output reg [3:0] hundred = 0 ); reg [3:0] i=0; reg [19:0] shift = 0; reg [3:0] temp_one =0; reg [3:0] temp_ten =0; reg [3:0] temp_hundred =0; reg [7:0] old_binary_value = 0; always@(posedge clk) begin if (i==0 & old_binary_value != binary_value) begin shift = 20'd0; old_binary_value = binary_value; shift[7:0] = binary_value; temp_hundred = shift[19:16]; temp_ten = shift[15:12]; temp_one = shift[11:8]; i = i + 1; end if(i < 9 & i > 0) begin if(temp_hundred >= 5) temp_hundred = temp_hundred + 3; if(temp_ten >= 5) temp_ten = temp_ten + 3; if(temp_one >= 5) temp_one = temp_one + 3; shift [19:8] = {temp_hundred , temp_ten , temp_one}; shift = shift << 1; temp_hundred = shift[19:16]; temp_ten = shift[15:12]; temp_one = shift[11:8]; i = i + 1; end if(i == 9) begin i = 0; hundred = temp_hundred; ten = temp_ten; one = temp_one; end end endmodule |
| --- | --- |

- RTL verification

- Gate-level verification

- Report

To convert the 8-bit binary input to BCD, we will first need an input clock and three outputs for the BCD digits (hundreds, tens, and ones). We will also create a register called "i" to count the eight cycles in the code, as well as five more registers: a 20-bit shift register to shift the input number, and registers for the hundreds, tens, and ones digits to keep the original value unchanged. Additionally, we will have one more register to keep the old binary input unchanged.

Starting with the code, we will set up three "if" statements. The first "if" statement checks if "i" is equal to 0 and if the old binary value is not equal to the new one. If this is the case, we will initialize the shift register to zero and set the old binary value to the new one. We will then set the shift register's [7:0] to the new binary value and set temporary hundreds to shift register [19:16], temporary tens to shift register [15:12], and temporary ones to shift register [11:8]. Finally, we will increment "i" by one.

The second "if" statement checks if "i" is less than nine and greater than zero. If this is the case, we will check if the hundreds digit is greater than four. If it is, we will add three to the hundred digits, and do the same for the tens and one digits. After adding three to each digit, we will store the result in the shift register.

Finally, the third "if" statement checks if "i" is equal to nine. If this is the case, we will set "i" back to zero, and assign temporary hundreds, tens, and ones to the corresponding registers.

BCD to seven segment display code converter

- Design

- Schematic entry

- Block diagram

- Behavioral

- Timing Verification

- Verilog-HDL entry

- Verilog entry

| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 | module segement7decoderverilog( input [3:0] bcd_in, output reg [7:0] seg_out ); always @ (bcd_in) begin case (bcd_in) 4'b0000: seg_out = 8'b0000001;//0 4'b0001: seg_out = 8'b1001111;//1 4'b0010: seg_out = 8'b0010010;//2 4'b0011: seg_out = 8'b0000110;//3 4'b0100: seg_out = 8'b1001100;//4 4'b0101: seg_out = 8'b0100100;//5 4'b0110: seg_out = 8'b0100000;//6 4'b0111: seg_out = 8'b0001111;//7 4'b1000: seg_out = 8'b0000000;//8 4'b1001: seg_out = 8'b0000100;//9 default: seg_out = 8'b0011111;//n endcase end endmodule |
| --- | --- |

- Verilog-HDL entry

- RTL verification

- Gate-level verification

- Report

To convert BCD to a seven-segment display, we first need an input for the BCD code and an output for the seven-segment display code. Next, because it needs to continuously receive signals, we use "always" to receive the input. Finally, we need to transfer the BCD code to the seven-segment display code. To do this, we can use a case statement to list the BCD codes and their corresponding seven-segment display codes. Here is what the case statement looks like:

| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 | always @(bcd_in) begin case (bcd_in) 4'b0000: seg_out = 8'b0000001; // 0 4'b0001: seg_out = 8'b1001111; // 1 4'b0010: seg_out = 8'b0010010; // 2 4'b0011: seg_out = 8'b0000110; // 3 4'b0100: seg_out = 8'b1001100; // 4 4'b0101: seg_out = 8'b0100100; // 5 4'b0110: seg_out = 8'b0100000; // 6 4'b0111: seg_out = 8'b0001111; // 7 4'b1000: seg_out = 8'b0000000; // 8 4'b1001: seg_out = 8'b0000100; // 9 default: seg_out = 8'b0011111; // blank endcase end |
| --- | --- |

Hardware costs

      - BCD to seven segment converter

| Logic gate | Number of costs(Schematic) |
| --- | --- |
| AND | 17 |
| OR | 18 |
| NOT | 4 |

      - Four bits full adder

| Logic gate | Number of costs(Schematic) |
| --- | --- |
| AND | 21 |
| NOT | 6 |
| NAND | 4 |
| NOR | 15 |
| XOR | 4 |

      - Geq5Plus3

| Logic gate | Number of costs(Schematic) |
| --- | --- |
| AND | 23 |
| OR | 2 |
| NOT | 6 |
| NAND | 4 |
| NOR | 15 |
| XOR | 4 |

      - Binary to BCD converter

| Logic gate | Number of costs(Schematic) |
| --- | --- |
| AND | 115 |
| OR | 10 |
| NOT | 30 |
| NAND | 20 |
| NOR | 75 |
| XOR | 20 |